WORLDWIDE INTEROPERABILITY

FOR MICROWAVE ACCESS (WIMAX)

1 Abstract

This paper presents the features of the Worldwide Interoperability for Microwave Access (WiMAX) technology and pretends to establish some valid criterions for future trends of possible applications of

WiMAX. A discussion is given by comparing Wireless Fidelity (Wi-Fi) and WiMAX. Several references have been included at the end of the article for those willing to know in detail about certain

specific topics.

2. Introduction

Broadband technology has rapidly become a need for all the population. Internet Service Providers (ISPs) have dealt with all sorts of challenges in order to deliver broadband solutions. In this sense, Digital

Subscriber Line (DSL) technology has appeared as a pioneer solution. However,coverage in wire line services is bounded and quality is another big issue. Wireless systems are an old solution that had been

displaced for its limits in bandwidth, LoS and the fact of delivering a cost effective solution.

Since a few years ago, when Wi-Fi was standardized and also its products regulated and certified by the Wi-Fi Forum, different solutions have come into

the market. Despite Wi-Fi was developed thinking in LAN solutions, it has been also used in MAN solutions but with a lot of limitations in its performance and certainly with trade-offs (bandwidth, coverage, power consumption).WiMAX is coming to fix this need anddeliver new broadband solutions for all theISPs and WIPSs that was harassed by the users’ needs of counting with more broadband for their different applications.

WiMAX is defined as Worldwide Interoperability for Microwave Access by the WiMAX Forum, formed in June 2001 to promote conformance and

interoperability of the IEEE 802.16 standard, officially known as Wireless MAN. The Forum describes WiMAX as "a standards-based technology enabling the delivery of last mile wireless broadband

access as an alternative to cable and DSL".

"WiMAX is not a technology, but rather a certification mark, or 'stamp of approval' given toequipment that meets certain conformity and interoperability tests for the IEEE 802.16 family of standards. A similar confusion surrounds the term Wi-Fi, which like WiMAX, is a certification mark for equipment based on a different set of IEEE standards from the 802.11 working group for wireless local area networks (WLAN).Neither WiMAX, nor Wi-Fi is a technology but their names have been adopted in popular usage to denote the technologies behind

them. This is likely due to the difficulty of using terms like 'IEEE 802.16' in common speech and writing."

3. 802.16 /HiperMAN

Technology Specs

- Based on IEEE 802.16 and ETSI

HiperMAN, WiMAX selected the

common mode of operation of

these two standards — 256FFT

OFDM.

- Concentrated in 2- to 11-GHz

WMAN, with the following set of

features:

- Service area range 50 km

- NLoS

- QoS designed in for

voice/video, differentiated

services

- Very high spectrum

utilization: 3.8 bit/Hz

- Up to 280 Mbps per BS

- Speed — 70 Mbps

- Defines both the MAC and PHY

layers and allows multiple PHY-layer

specifications

4. WiMAX—Evolution of the

Technology

As the envisioned usage scenario has evolved over time, so has evolved the technological basis of WiMAX. The IEEE 802.16 technical specification has now evolved through three generations:

• IEEE 802.16: High data rate, highpower,PTP, LOS, fixed SSs

• IEEE 802.16-2004: Medium data rate, PTP, PMP, fixed SSs

• IEEE 802.16-2005: Low-medium data rate, PTP, PMP, fixed or mobile SSs

5. WiMAX System

A WiMAX system consists of two parts:

• A WiMAX tower, similar in concept to  a cell-phone tower

- A single WiMAX tower can provide coverage to a very large area -- as big as 3,000 square miles (~8,000square km).

• A WiMAX receiver – The receiver and antenna could be a small box or PCMCIA card, or they could be built into a laptop the way WiFi access is today.

A WiMAX tower station can connect directly to the Internet using a highbandwidth, wired connection (for example,a T3 line). It can also connect to another

WiMAX tower using a line-of-sight,microwave link. This connection to a second tower (often referred to as a backhaul), along with the ability of a single tower to cover up to 3,000 square miles, is what allows WiMAX to provide coverage to remote rural areas.

Compared to the complicated wired network, a WiMAX system only consists of two parts:

The WiMAX base station (BS) and WiMAX subscriber station (SS), also referred to as customer premise equipments (CPE). Therefore, it can be

built quickly at a low cost. Ultimately,WiMAX is also considered as the next step in the mobile technology evolution path.The potential combination of WiMAX and CDMA standards is referred to as 4G.

5.1 System Model

IEEE 802.16 supports two modes of

operation: PMP and PTP.

5.1.1 Point-to-point (PTP)

The PTP link refers to a dedicated link that

connects only two nodes: BS and

subscriber terminal. It utilizes resources in

an inefficient way and substantially causes

high operation costs. It is usually only

used to serve high-value customers who

need extremely high bandwidth, such as business high-rises, video postproduction houses, or scientific research organizations. In these cases, a single

connection contains all the available bandwidth to generate high throughput. A highly directional and high-gain antenna is also necessary to minimize interference and maximize security.

5.1.2 Point-to-multipoint (PMP)

The PMP topology, where a group of subscriber terminals are connected to a BS separately (shown in Figure), is a better choice for users who do not need to use the entire bandwidth. Under PMP topology, sectoral antennas with highly directional

parabolic dishes (each dish refers to a sector) are used for frequency reuse. The available bandwidth now is shared between a group of users, and the cost for each subscriber is reduced.

6. WiMAX as a Metro-Access Deployment Option

WiMAX is a worldwide certification addressing interoperability across IEEE 802.16 standards-based products. The IEEE 802.16 standard with specific

revisions addresses two usage models:

- Fixed

- Portable

6.1 Fixed

The IEEE 802.16-2004 standard (which revises and replaces IEEE 802.16a and 802.16REVd versions) is designed for fixed-access usage models. This standard may be referred to as “fixed wireless”

because it uses a mounted antenna at the

subscriber’s site.

6.2 Portable

The IEEE 802.16e standard is an amendment to the 802.16-2004 base specification and targets the mobile market by adding portability and the ability for mobile clients with IEEE 802.16e adapters to connect directly to the WiMAX network to the standard. The 802.16e standard is expected to be ratified in early 2005.

7. WiMAX Physical Layer

The WiMAX physical layer is based on orthogonal frequency division multiplexing. OFDM is the transmission scheme of choice to enable high-speed data, video, and multimedia communications and is used by a variety of commercial broadband systems, including DSL, Wi-Fi, Digital Video Broadcast-Handheld (DVB-H), and MediaFLO, besides WiMAX. OFDM is an elegant and efficient scheme for high data rate transmission in a non-line-of-sight or multipath radio environment.

7.1 OFDM Technology

Orthogonal frequency division multiplexing (OFDM) technology provides operators with an efficient means to overcome the challenges of NLOS propagation. OFDM is based on the traditional frequency division multiplexing (FDM), which enables simultaneous transmission of multiple signals by separating them into different frequency bands (subcarriers) and sending them in parallel. In FDM, guard bands are needed to reduce the interference between different frequencies, which causes bandwidth wastage. Therefore, it is not a spectrum-efficient and cost-effective solution. However, OFDM is a more spectrum-efficient method that removes all the guard bands but keeps the modulated signals orthogonal to mitigate the interference level.

Comparison between FDM and OFDMA

As shown in figure the required bandwidth in OFDM is significantly decreased by spacing multiple modulated carriers closer until they are actually overlapping.OFDM uses fast Fourier transform (FFT) and inverse FFT to convert serial data to multiple channels. The FFT size is 256,which means a total number of 256 subchannels (carriers) are defined for OFDM. In OFDM, the original signal is divided into 256 subcarriers and transmitted in parallel. Therefore, OFDM is referred to as a multicarrier modulation

scheme. Compared to single-carrier schemes, OFDM is more robust against multipath propagation delay owing to the use of narrower subcarriers with low bit rates resulting in long symbol periods.A guard time is introduced at each OFDM symbol to further mitigate the effect of multipath delay spread. The WiMAX OFDM waveform offers the advantage of being able to operate with the larger delay spread of the NLOS environment. By virtue of the OFDM symbol time and use of a cyclic prefix, the OFDM waveform eliminates the inter-symbol interference (ISI) problems and the complexities of adaptive equalization. Because the OFDM waveform is composed of multiple narrowband orthogonal carriers, selective fading is localized to a subset of carriers that are relatively easy to equalise. An example is shown below as a comparison between an OFDM signal and a single carrier signal, with the information being sent in parallel for OFDM and in series for single carrier.

The ability to overcome delay spread, multi-path, and ISI in an efficient manner allows for higher data rate throughput. As an example it is easier to equalize the

individual OFDM carriers than it is to equalize the broader single carrier signal.

For all of these reasons recent international standards such as those set by IEEE 802.16, ETSI BRAN, and ETRI, have established OFDM as the preferred

technology of choice.

7.2 OFDM Parameters in

WiMAX

As mentioned previously, the fixed and mobile versions of WiMAX have slightly different implementations of the OFDM physical layer. Fixed WiMAX, which is based on IEEE 802.16- 2004, uses a 256 FFT-based OFDM physical layer. Mobile WiMAX, which is based on the IEEE 802.16e-20055 standard, uses a scalable OFDMA-based physical layer. In the case of mobile WiMAX, the FFT sizes can vary from 128 bits to 2,048 bits.

7.2.1 Fixed WiMAX OFDM-PHY:

For this version the FFT size is fixed at 256, which 192 subcarriers used for carrying data, 8 used as pilot subcarriers for channel estimation and synchronization purposes, and the rest used as guard band subcarriers.

7.2.2 Mobile WiMAX OFDMA-PHY: In

Mobile WiMAX, the FFT size is scalable from 128 to 2,048. Here, when the available bandwidth increases, the FFT size is also increased such that the subcarrier spacing is always 10.94kHz.

7.3 Sub Channelization OFDMA

Sub Channelization in the uplink is an option within WiMAX. Sub channeling enables the link budget to be balanced such that the system gains are similar for both the up and down links. Sub channeling concentrates the transmit power into fewer OFDM carriers;this is what increases the system gain that can either be used to extend the reach of the system, overcome the building

penetration losses, and or reduce the power

consumption of the CPE. The use of sub

channeling is further expanded in

orthogonal frequency division multiple

access (OFDMA) to enable a more flexible

use of resources that can support nomadic

or mobile operation.

7.4 MAC-Layer Overview

The primary task of the WiMAX MAC layer is to provide an interface between the higher transport layers and the physical layer. The MAC layer takes packets from the upper layer—these packets are called MAC service data units (MSDUs)—and organize them into MAC protocol data units (MPDUs) for transmission over the air. For received transmissions, the MAC layer does the reverse. The IEEE 802.16- 2004 and IEEE 802.16e-2005 MAC design includes a convergence sub layer that can interface with a variety of higher-layer protocols, such as ATM, TDM Voice, Ethernet, IP, and any unknown future protocol.

7.5 Power Control

Power control algorithms are used to improve the overall performance of the system, it is implemented by the base station sending power control information to each of the CPEs to regulate the transmit power level so that the level received at the base station is at a predetermined level. In a dynamical changing fading environment this pre-determined performance level means that the CPE only transmits enough power to meet this requirement. The converse would be that the CPE transmit level is based on worstcase

conditions. The power control reduces the overall power consumption of the CPE and the potential interference with other co-located base stations. For LOS the transmit power of the CPE is approximately proportional to it’s distance from the base station, for NLOS it is also heavily dependant on the clearance and obstructions.

7.6 Adaptive Modulation

Adaptive modulation allows the WiMAX system to adjust the signal modulation scheme depending on the signal to noise ratio (SNR) condition of the radio link.When the radio link is high in quality, the highest modulation scheme is used, giving the system more capacity. During a signal fade, the WiMAX system can shift to a lower modulation scheme to maintain the connection quality and link stability. This feature allows the system to overcome time-selective fading. The key feature of adaptive modulation is that it increases the range that a higher modulation scheme can be used over, since the system can flex to the actual fading conditions, as opposed to having a fixed scheme that is budgeted for the worst case conditions.

7.7 Error Correction Techniques

Error correction techniques have been incorporated into WiMAX to reduce the system signal to noise ratio requirements.Strong Reed Solomon FEC, convolutional encoding, and interleaving algorithms are used to detect and correct errors to improve throughput. These robust error correction techniques help to recover errored frames that may have been lost due to frequency selective fading or burst errors. Automatic repeat request (ARQ) is used to correct errors that cannot be corrected by the FEC, by having the errored information resent. This significantly improves the bit error rate (BER) performance for a similar threshold level.

8. Competing technologies

Within the marketplace, WiMAX’s main competition comes from widely deployed wireless systems with overlapping functionality such as UMTS and CDMA2000, as well as a number of Internet-oriented systems such as HIPERMAN and WiBro . Both of the two major 3G systems, CDMA2000 and UMTS, compete with WiMAX. Both offer DSL-class Internet access, in addition to phone service. UMTS has also been enhanced to compete directly with WiMAX in the form of UMTS-TDD, which can use WiMAX-oriented spectrum, and it provides a more consistent (lower bandwidth at peak) user experience than

WiMAX (Figure). Moving forward, similar air interface technologies to those used by WiMAX are being considered for the 4G evolution of UMTS .

8.1 WiBro

WiBro (wireless broadband) is an Internet

technology being developed by the Korean telecom industry (Figure). In February 2002, the Korean government allocated 100 MHz of electromagnetic spectrum in the 2.3-GHz band, and in late 2004, WiBro Phase 1 was standardized by the TTA (Telecommunications Technology Association) of Korea. WiBro is the newest variety of mobile wireless broadband access. It is based on the same IEEE 802.16 standard as WiMAX but is designed to maintain connectivity on the go, tracking a receiver at speeds of up to 37 mi per hr (60 km/hr). WiMAX is the current standard in the United States, offering wireless Internet connectivity to mobile users at fixed ranges of up to 31 mi

(50 km) from the transmitting base. However, it is not designed to be used while the receiver is in motion. WiBro can be thought of as mobile WiMAX, though the technology and its exact specifications will change as it undergoes refinements throughout its preliminary stages.

9. Advantages over Wi-Fi

The WiMAX specification provides symmetrical bandwidth over many kilometers and range with stronger encryption (TDES or AES) and typically less interference. Wi-Fi is short range (approximately 10's of metres) has WEP or WPP encryption and suffers from interference as in metropolitan areas where there are many users.Wi-Fi Hotspots are typically backhauled over ADSL in most

coffee shops therefore Wi-Fi access is typically highly contended and has poor upload speeds between the router and the internet.It provides connectivity between network endpoints without the need for direct line of sight in favourable circumstances. The non-line-of-sight propagation (NLOS) performance requires the .16d or .16e revisions, since the lower frequencies are needed. It relies upon multi-path signals, somewhat in the manner of 802.11n.

9.1 Benefits of WiMAX

Component Suppliers

• Assured wide market acceptance of developed and components

• Lower production costs due to economies of scale

• Reduced risk due to interoperability

Equipment Manufacturers

• Stable supply of lowcost components and chips

• Engineering development efficiencies

• Lower production costs due to economies

of scale.

Operators and Service Providers

• Lower CAPEX – with lower cost base station, customer premises equipment (CPE), and network deployment costs

• Lower investment risk due to freedom of

choice among multiple vendors and solutions

• Improved operator business case with

lower OPEX.

End Users

• Lower subscriber fees

• Portability of terminals when moving locations/networks from WiMAX operator “A” to operator “B”

• Lower service rates over time due to cost

efficiencies in the delivery chain

• Wider choice of terminals enabling cost performance analysis

10. Limitations

A commonly-held misconception is that WiMAX will deliver 70 Mbit/s over 50 kms. In reality, WiMAX can do one or the other — operating over maximum range (50 km) increases bit error rate and thus must use a lower bitrate. Lowering the range allows a device to operate at higher bitrates.Typically, fixed WiMAX networks have a higher-gain directional antenna installed near the client (customer) which results in greatly increased range and throughput.Mobile WiMAX networks are usually made of indoor "customer premises equipment" (CPE) such as desktop modems, laptops with integrated Mobile WiMAX or other Mobile WiMAX devices. Mobile WiMAX devices typically have an omni-directional antenna which is of lower-gain compared to directional antennas but are more portable. In practice, this means that in a line-of-sight environment with a portable Mobile WiMAX CPE, speeds of 10 Mbit/s at 10 km could be delivered However, in urban environments they may not have line-of sight and therefore users may only receive 10 Mbit/s over 2 km. Higher-gain directional antennas can be used with a Mobile WiMAX network with range and throughput benefits but the obvious loss of practical mobility.

11. Future of WiMAX

11.1 The IEEE 802.20 Standard

The IEEE 802.20 standard is a broadband wireless networking technology that is being standardized for deployment by mobile communications service providers,in portions of their licensed spectrum. The

capacity of 802.20 is projected to be 2 Mbps per user, and its range is comparable to 3G cellular technologies,namely, up to 5 km. More typical deployments will be in the neighborhood of 1 to 3km. Finalization of the 802.20 standard is not expected soon. The 802.20 standard has been under development since late 2002,but the going has been slow, to say the least. 802.20 and 802.16e, the mobile WiMAX specification, appear similar at first glance but differ in the frequencies they will use and the technologies they are based on. Standard 802.20 will operate below 3.5 GHz, whereas mobile WiMAX will work within the 2-GHz to 6-GHz bands. Further, as the name suggests,802.16e is based on WiMAX, with the goal of having WiMAX transmitters being able to support both fixed and mobile connections. Although the 802.20 group will be back at work later, the 802.20 technology is alluring, with promises of low-latency 1-Mbps connections being sustained even at speeds of up to 150 mph,but we are going to have to wait a couple of years for it.

12. CONCLUSIONS

It is expected that WiMAX becomes the dominant standard for Wireless MAN networks in the world market, at least, in fixed broadband networks. A brief

comparison between 802.16 and 802.16a has been provided and also it has been shown the advantage by using adaptive modulation. It has been explained that the key difference between the initial 802.16 standard and the 802.16a consists of the modulation scheme.

The importance of OFDM has also been analyzed and this becomes an important feature that makes the difference between the 802.16 and 802.16a standard. More about this topic can be found in the literature provided. PHY and MAC layers of WiMAX have been discussed Future possible applications have been discussed. WiMAX mobility standard is the next step. However, it will have its competition too with the 802.20 standard that in short is called Mobility-Fi. We will have to wait for the products and their performance in real environments in order to evaluate what the standard addresses and the real performance of these products. There are already prototypes and also development kits using WiMAX standard that are used for education and mainly for research. Nowadays, there are also some products that have been introduced into the market that already contains the WiMAX standard presented here.

Market is the key word to take into account. products will have to be delivered according to the market needs and those for end-users will have to be extremely easy to install. Experience from DSL and

cable modems services shows this drawback. Of course, in addition to be easy to install and provide good technical features, these products have to provide low-cost or at least a clear advantage over

other technologies that are, at this moment,already matured in the market like xDSL and cable modem.

13. References

1. IEEE 802.16-2001, IEEE standard for local and metropolitan area networks—Part 16: Air interface

for fixed broadband wireless access systems, 6 December 2001.

2. IEEE 802.16a-2001, IEEE standard for local and metropolitan area networks—Part 16: Air interface

for fixed broadband wireless access systems—Amendment

2: Medium access control modifications and additional physical layer specifications for 2–11 GHz, 1 April 2003.

3. http://www.wirelessdesignasia.com /article.asp?id=2049.

4.http://www.intel.com/netcomms/events/wimax.htm.

5.http://www.btimes.com.my/Current_News/BT/Monday/Column/BT58 3229.txt/Article/ [dated August 24,

2006].

6. www.wimaxforum.org.

7.http://www.qoscom.de/documentation/51_WiMAX%20Summit%20paris%20-%20may04.pdf.

8. The Implications of WiMax for Competition and Regulation,OECD document [dated March 2,

2006].

9.http://ww.sfgate.com/cgibin/article.cgi?f=/c/a/2006/12/18/BUG8NN0HIT1.DTL [dated December 18, 2006].

10. http://electronicxtreme.blogspot.com/2006/12/wi-max.html [dated December 11, 2006].

Key Terminology

BS –  Base station

DSL –  Digital subscriber line

ETSI –  European Telecommunications Standards

Institute

FCC –  Federal Communications Commission

IEEE –  Institute of Electrical and Electronics

Engineers

IP – Internet Protocol

LAN – Local area network

MAC address – Media access control address. This

address is a computer’s unique hardware number.

MAN – Metropolitan area network

OFDM – Orthogonal frequency division

multiplexing

OFDMA – Orthogonal frequency division-multiple

access

P2P – Point-to-point

P2MP – Point-to-multi-point

PAN – Personal area network

PHY – Physical layer

PoP – Point of presence

QoS – Quality of service

RF – Radio frequency

SS – Subscriber station

UWB – Ultra-wide band

VoIP – Voice over Internet Protocol

WAN – Wide area network

Wi-Fi – Wireless fidelity. Used generically when

referring to any type of 802.11 network, whether 802.11b,

802.11a,dual-band, and so on.

WiMAX – Worldwide Interoperability for Microwave Access

WISP – Wireless Internet service provider

WLAN – Wireless local area network

WMAN – Wireless metropolitan area network

WWAN – Wireless wide area networks